UNIVERSAL TIMER	PSEUDO CODE

init

dispZone    16 bytes

mode	0	time

1..14	prog

15,16	light prog

- set

timeSetting 0 | 1

cursor	0..15

relay 	0 off

1 on

2 random

snooze	0..9

timesetting = 1

cursor = 0

…

loadFromEeprom

return

main loop

ReadAD0

keyboardRead

if keys = 0, 0 -> busy flag in kbd control flags

if not busy flag, verwerkKbd

case mode:

0:	showDispTime

1..14:	showDispProg

15,16:	showDispProgLight

17:	showDispSet

evaluate

showRelay

showDispZone

Loop

verwerkKbd

if keys = 0, return

1 -> busy flag in kbd control flags

case mode:

0:	kbdTime

1..14:	kbdProg

15,16:	kbdProgLight

17:	kbdSet

return

kbdProg

case keys:

1:	mode = 0

store2Eeprom

return

2:	incrMode

cursor = 0

return

3:	incrCursor

return

4:	fetchProgData(mode)

case cursor:

0:	ProgDataH -> hourVar

addHour10

hourVar -> ProgDataH

1:	likewise

3:	likewise

4:	likewise

6:	ProgDataD -> dayVar

addDay1

dayVar -> ProgDataD

9:	ProgDataA -> actionVar

addAction1

actionVar -> ProgDataA

storeProgData(mode)

return

showDispTime

Hours -> dispZone

Minutes -> dispZone

Seconds -> dispZone

Day -> dispZone

return

showDispProg

fetchProgData(mode)

ProgDataH -> dispZone

ProgDataM -> dispZone

ProgDataD-> dispZone

ProgDataA -> dispZone

return

showDispZone

if timeSetting = 1 or mode != 0,

if ptHalfSecond (in show flags),

dispZone[cursor] = ‘ ‘

if ptSecond (in show flags),

0 -> ptSecond, ptHalfSecond (in show flags)

dispZone -> LCD

return

kbdTime

case keys:

1:	if timeSetting = 1, 0 -> timeSetting

return

2:	if timeSetting = 0, incrMode

return

3:	if timeSetting = 1, incrCursor, else incrRelay

return

4:	if timeSetting = 0, 9 -> snooze and return

case cursor:

0:	Hours -> hourVar

addHour10

hourVar -> Hours

1:	hours -> hourVar

addHour1

hourVar -> Hours

3:	Minutes -> minuteVar

addMinute10

minuteVar -> Minutes

4:	Minutes -> minuteVar

addMinute1

minuteVar -> Minutes

6:	0 -> Seconds

9:	Days++ and check limits

return

evaluate

light 	0 | 1

prevLight

Hours1516

evaluateLight

loop through all 14 timer progs checking

each action, if action != 3 check hours, minutes

and day, if match, action -> relay

if at least one match, return

if Hours1516 = 0,

prevRelay -> relay

Hours1516 = 0xFE

Return

if light = prevLight, return

light -> prevLight

update yellowLed according to light

relay -> prevRelay

if light,

OnDarkH -> Hours1516

if OnDarkA = 3, 0xFE -> Hours1516

else

OnLightH -> Hours1516

if OnLightA = 3, 0xFE -> Hours1516

clear MinuteCounter

if light,

if OnLightA != 3, OnLightA -> relay

else

if OnDarkA != 3, OnDarkA -> relay

return